SAMPLE — CRITERIA NOT VERIFIED Thresholds marked * are PLACEHOLDERS, not from the AFM or ops specs. Layout sample only — not for dispatch planning.

AIRPORT SUITABILITY ASSESSMENT

KDTO — Denton Enterprise Airport, Denton, TX

| DATA SOURCE | NASR CYCLE | ASSESSED BY | DATE |
| --- | --- | --- | --- |
| FAA NASR | 2026-07-09 (sample) | D. Holsapple | 14 Jul 2026 |

1.  DETERMINATION

| AIRCRAFT | DETERMINATION | QUALIFYING RUNWAYS |
| --- | --- | --- |
| Falcon 50EX | SUITABLE | 18L/36R |
| Gulfstream G280 | SUITABLE WITH LIMITATIONS | 18L/36R only |

G280: single qualifying runway. If 18L/36R is out of service the airport does not meet criteria. See §7.

2.  FACILITY

From FAA NASR. Authoritative.

| CONTROL TOWER | YES — 0600-2200 L | ELEVATION | 642 ft MSL |
| --- | --- | --- | --- |
| AIRPORT ATTENDANCE | CONTINUOUS | ARFF INDEX | NOT IN NASR |
| FUEL ON FIELD | Jet A, 100LL | AIRPORT MANAGER | 940-349-7744 |

NASR REMARKS

• RWY 18L designated as calm wind runway. • For clearance delivery when ATCT is closed, contact Lone Star Approach at 972-615-2799. • Dusk-dawn when ATCT closed: MIRL 18L/36R and 18R/36L preset low intensity. To increase intensity and activate MALSR RWY 18L — use CTAF. • Airport closed to ultralights and gliders. Mowing operations May-Sep.

3.  COMMUNICATIONS

| FACILITY | FREQUENCY | NOTES |
| --- | --- | --- |
| Tower | 119.95 | 0600-2200 L |
| Ground | 123.95 | 0600-2200 L |
| Clearance Delivery | 123.95 | 0600-2200 L |
| Approach / Departure | 118.1 | Lone Star Approach |
| ATIS / ASOS | 119.325 | Phone 940-383-8457 |
| CTAF | 119.95 | When tower closed |
| CD when ATCT closed | — | Phone Lone Star Approach 972-615-2799 |

4.  RUNWAY EVALUATION — ALL RUNWAYS

| RUNWAY | LENGTH | WIDTH | SURFACE | WT BRG | 50EX | G280 |
| --- | --- | --- | --- | --- | --- | --- |
| 18L/36R | 7,002 ft | 150 ft | Asphalt | S-70 D-100 | PASS (+1,502 ft) | PASS (+1,002 ft) |
| 18R/36L | 5,003 ft | 75 ft | Asphalt | S-30 | FAIL (−497 ft) | FAIL (−997 ft) |

5.  CRITERIA EVALUATION — FALCON 50EX / RWY 18L/36R

| CRITERION | ACTUAL | REQUIRED | MARGIN | RESULT |
| --- | --- | --- | --- | --- |
| Runway length | 7,002 ft | 5,500 ft * | +1,502 ft | PASS |
| Runway width | 150 ft | 75 ft * | +75 ft | PASS |
| Weight bearing (DW) | 100,000 lb | 40,780 lb * | +59,220 lb | PASS |
| Surface | Asphalt | Paved | — | PASS |
| Runway lighting | MIRL + MALSR | Required | — | PASS |
| ARFF index | NOT IN NASR | Index B * | — | UNKNOWN |
| Approach minimums | 200-1/2 | ≤ 400 ft * | +200 ft | PASS |
| Alternate distance | KDFW 24 NM | ≤ 50 NM * | +26 NM | PASS |

*  Placeholder threshold — replace with AFM / ops spec value.

6.  SERVICES AND FBO

NOT FROM NASR — CONFIRM BY PHONE. FBO services, hours, and fuel change without notice. These entries are a starting point and a phone number, not a verified fact.

| FBO | PHONE | FUEL | SERVICES |
| --- | --- | --- | --- |
| Sheltair DTO | 940-435-2621 | Jet A, 100LL | Hangar, crew car, Hertz on site, catering, cargo, lounge |
| US Trinity Aviation | CONFIRM | Jet A | Fuel, hangar, ground handling, concierge |
| Jet Works Air Center | 940-323-8700 | CONFIRM | Maintenance |

7.  DISQUALIFYING FINDINGS

| RWY 18R/36L | 5,003 ft below minimum for both airframes. Not usable by either aircraft. |
| --- | --- |
| G280 — airport | Single qualifying runway. No usable alternate runway on field if 18L/36R closes. |

8.  ITEMS REQUIRING VERIFICATION

Auto-generated. Every data point the source could not supply becomes an action item.

| # | ITEM | ACTION |
| --- | --- | --- |
| 1 | ARFF index not published in NASR | Confirm with airport ops — 940-349-7744 |
| 2 | Grooved surface status not in NASR | Verify via AFD or airport ops |
| 3 | US Trinity Aviation phone not confirmed | Look up and confirm before dispatch |
| 4 | Jet Works fuel availability not confirmed | Call 940-323-8700 |

9.  PIC NARRATIVE

10.  SIGN-OFF

| ROLE | NAME | SIGNATURE | DATE |
| --- | --- | --- | --- |
| Pilot in Command |  |  |  |
| Chief Pilot |  |  |  |

This assessment is a planning guide. The Pilot in Command determines actual airport and runway usability for the conditions at the time of operation.